Day3 TLVERILOG ASSIGNMENTS:

Combinational calculator using TLVERILOG:

Code:

\m4_TLV_version 1d: tl-x.org

\SV

// =========================================

// Welcome!  Try the tutorials via the menu.

// =========================================

// Default Makerchip TL-Verilog Code Template

// Macro providing required top-level module definition, random

// stimulus support, and Verilator config.

m4_makerchip_module   // (Expanded in Nav-TLV pane.)

\TLV

$reset = *reset;

$val1[31:0]  = $rand1[3:0];

$val2[31:0]  = $rand2[3:0];

$sum[31:0]   = $val1 + $val2;

$sub[31:0]   = $val1 - $val2;

$mul[31:0]   = $val1 * $val2;

$quotient[31:0] = $val1 / $val2;

$out[31:0] = $op[1]? ($op[0]? ($sum) : ($sub)): ($op[0]? ($mul) : ($quotient));

// Assert these to end simulation (before Makerchip cycle limit).

*passed = *cyc_cnt > 40;

*failed = 1'b0;

\SV

endmodule

SEQUENTIAL CIRCUITS: [implementation is something like below]

Next stage of any flop is obtained by applying combinational logic on previous stage.

Counter:  combo logic for counter contains one adder and a [2:1] mux, selection line of mux is reset

>>1$num in the code means the datacapturedafter one clockadvance

Code:

\m4_TLV_version 1d: tl-x.org

\SV

// Macro providing required top-level module definition, random

// stimulus support, and Verilator config.

m4_makerchip_module   // (Expanded in Nav-TLV pane.)

\TLV

$reset = *reset;

$num[31:0] = $reset? 0 :  (>>1$num+1);

//...

// Assert these to end simulation (before Makerchip cycle limit).

*passed = *cyc_cnt > 40;

*failed = 1'b0;

\SV

endmodule

Sequential calculator:

The data gets copied from out to val1 in next clock cycle.

Code:

\m4_TLV_version 1d: tl-x.org

\SV

// =========================================

// Welcome!  Try the tutorials via the menu.

// =========================================

// Default Makerchip TL-Verilog Code Template

// Macro providing required top-level module definition, random

// stimulus support, and Verilator config.

m4_makerchip_module   // (Expanded in Nav-TLV pane.)

\TLV

$reset = *reset;

$val1[31:0]  =  >>1$out;

// $val1[31:0]  = $rand1[3:0];

$val2[31:0]  = $rand2[3:0];

$sum[31:0]   = $val1 + $val2;

$sub[31:0]   = $val1 - $val2;

$mul[31:0]   = $val1 * $val2;

$quotient[31:0] = $val1 / $val2;

$out[31:0] = $reset? 0 :  ($op[1]? ($op[0]? ($sum) : ($sub)): ($op[0]? ($mul) : ($quotient)));

// Assert these to end simulation (before Makerchip cycle limit).

*passed = *cyc_cnt > 40;

*failed = 1'b0;

\SV

endmodule

combo logic:

Counterwithpipeline:

Code:

m4_makerchip_module   // (Expanded in Nav-TLV pane.)

\TLV

|count //pipe

@0   //stage

$reset = *reset;

@1

$num[31:0] = $reset? 0 :  (>>1$num+1);

//...

// Assert these to end simulation (before Makerchip cycle limit).

*passed = *cyc_cnt > 40;

*failed = 1'b0;

\SV

endmodule

Counter+calculatorwithsinglecyclepipeline:

\m4_TLV_version 1d: tl-x.org

\SV

// =========================================

// Welcome!  Try the tutorials via the menu.

// =========================================

// Default Makerchip TL-Verilog Code Template

m4_include_lib //(['https://raw.githubusercontent.com/stevehoover/RISC-V_MYTH_Workshop/bd1f186fde018ff9e3fd80597b7397a1c862cf15/tlv_lib/calculator_shell_lib.tlv'])

\SV

// Macro providing required top-level module definition, random

// stimulus support, and Verilator config.

m4_makerchip_module   // (Expanded in Nav-TLV pane.)

//m4+cal_viz(@3)

\TLV

|calc

@0

$reset = *reset;

@1

$val1[31:0]  =  >>1$out;

// $val1[31:0]  = $rand1[3:0];

$val2[31:0]  = $rand2[3:0];

$op[1:0]    = $rand1[1:0];

$sum[31:0]   = $val1 + $val2;

$sub[31:0]   = $val1 - $val2;

$mul[31:0]   = $val1 * $val2;

$quotient[31:0] = $val1 / $val2;

$out[31:0]  = $reset? 0 :  ($op[1]? ($op[0]? ($sum) : ($sub)): ($op[0]? ($mul) : ($quotient)));

$count[31:0] = $reset? 0 :  (>>1$count+1);

m4+cal_viz(@3) // Arg: Pipeline stage represented by viz, should be atleast equal to last stage of CALCULATOR logic.

// Assert these to end simulation (before Makerchip cycle limit).

*passed = *cyc_cnt > 40;

*failed = 1'b0;

\SV

endmodule

2:calculator with double cycle pipeline

Code:

\m4_TLV_version 1d: tl-x.org

\SV

// =========================================

// Welcome!  Try the tutorials via the menu.

// =========================================

// Default Makerchip TL-Verilog Code Template

m4_include_lib(['https://raw.githubusercontent.com/stevehoover/RISC-V_MYTH_Workshop/bd1f186fde018ff9e3fd80597b7397a1c862cf15/tlv_lib/calculator_shell_lib.tlv'])

\SV

// Macro providing required top-level module definition, random

// stimulus support, and Verilator config.

m4_makerchip_module   // (Expanded in Nav-TLV pane.)

//m4+cal_viz(@3)

\TLV

|calc

@0

$reset = *reset;

@1

$val2[31:0]  = $rand2[3:0];

// $op[1:0]     = $rand1[1:0];

$valid = $reset? 0 :  (>>1$valid+1);

$val1[31:0]  =  >>2$out;

// $val1[31:0]  = $rand1[3:0];

$sum[31:0]   = $val1 + $val2;

$sub[31:0]   = $val1 - $val2;

$mul[31:0]   = $val1 * $val2;

$quotient[31:0] = $val1 / $val2;

@2

$valid_reset = ($reset || (!($valid)));

$out[31:0]  = $valid_reset ? 0 : ($op[1] ? ($op[0] ? $sum : $sub): ($op[0] ? $mul : $quotient ));

// m4+cal_viz(@2) // Arg: Pipeline stage represented by viz, should be atleast equal to last stage of CALCULATOR logic.

// Assert these to end simulation (before Makerchip cycle limit).

*passed = *cyc_cnt > 40;

*failed = 1'b0;

\SV

endmodule

Calculator with single value memory:

Code:

\m4_TLV_version 1d: tl-x.org

\SV

// =========================================

// Welcome!  Try the tutorials via the menu.

// =========================================

// Default Makerchip TL-Verilog Code Template

m4_include_lib(['https://raw.githubusercontent.com/stevehoover/RISC-V_MYTH_Workshop/bd1f186fde018ff9e3fd80597b7397a1c862cf15/tlv_lib/calculator_shell_lib.tlv'])

\SV

// Macro providing required top-level module definition, random

// stimulus support, and Verilator config.

m4_makerchip_module   // (Expanded in Nav-TLV pane.)

//m4+cal_viz(@3)

\TLV

|calc

@0

$reset = *reset;

@1

$val2[31:0]  = $rand2[3:0];

// $op[1:0]     = $rand1[1:0];

$valid = ($reset)? 0 :  (>>1$valid+1);

$val1[31:0]  =  >>2$out;

// $val1[31:0]  = $rand1[3:0];

$sum[31:0]   = $val1 + $val2;

$sub[31:0]   = $val1 - $val2;

$mul[31:0]   = $val1 * $val2;

$quotient[31:0] = $val1 / $val2;

@2

$valid_reset = ($reset || (!($valid)));

$mem[31:0]   = ($reset)?  0 : (($op[2:0] == 3'b101)?  $val1 : >>2$mem) ;

$out[31:0]   =  ($reset)? 32'b0 :

($op[2:0] == 3'b000)? $quotient :

($op[2:0] == 3'b001)? $mul :

($op[2:0] == 3'b010)? $sub :

($op[2:0] == 3'b011)? $sum :

($op[2:0] == 3'b100)? >>2$mem : >>2$out ;

m4+cal_viz(@2) // Arg: Pipeline stage represented by viz, should be atleast equal to last stage of CALCULATOR logic.

// Assert these to end simulation (before Makerchip cycle limit).

*passed = *cyc_cnt > 40;

*failed = 1'b0;

\SV

endmodule